Final Project Report: SysTec Smart Thermostat

Luca Formento

My Raspberry Pi 4B prototype meets SysTec's low-level requirements: it reads temperature from the AHT20 over I2C, drives the status LEDs and three buttons over GPIO, updates the 16x2 LCD, and sends a status string over the UART every 30 seconds. For the Wi-Fi production phase, I compared the Raspberry Pi Zero 2 W, the Microchip WFI32E01, and the NXP RW612 (Table 1).

Table 1. Architecture comparison

| Requirement | Raspberry Pi Zero 2 W | Microchip WFI32E01 | NXP RW612 |
| --- | --- | --- | --- |
| Processor | Quad-core Cortex-A53, 1 GHz | PIC32MZ W1, 200 MHz MIPS32 | Cortex-M33, 260 MHz |
| Wi-Fi | 2.4 GHz 802.11 b/g/n | 2.4 GHz 802.11 b/g/n, WPA3 | Wi-Fi 6, plus BLE 5.3 |
| Project peripherals | I2C, UART, SPI, GPIO, PWM on the same 40-pin header | 1 I2C, 3 UART, 2 SPI, GPIO | I2C, USART, SPI, GPIO, PWM |
| Memory | 512 MB RAM, microSD storage | 1 MB flash, 256 KB RAM | 1.2 MB SRAM, external flash |
| Runs current Python code | Yes, unchanged | No, port to C | No, port to C |

Supporting the Project Peripherals

All three support the project's peripherals. The Pi Zero 2 W uses the same 40-pin header as the Pi 4B, so the sensor, LCD, LEDs, buttons, and UART connect to the same pins with the same libraries. The WFI32E01 offers one I2C port, three UARTs, and two SPI ports (Mouser Electronics, n.d.), enough for this design, but the LED and button logic would be rewritten in C. The RW612 lists I2C, USART, SPI, GPIO, and hardware PWM for the fading LEDs (Zephyr Project, n.d.).

Connecting to the Cloud via Wi-Fi

All three have integrated Wi-Fi. The Pi Zero 2 W has 802.11 b/g/n on board (SparkFun Electronics, n.d.), and since it runs Linux, my UART status line could become an MQTT or HTTPS message using standard Python libraries. The WFI32E01 supports WPA3 and TLS and includes a Trust&GO secure element pre-provisioned for AWS, Azure, and Google Cloud (Mouser Electronics, n.d.), which I think is its biggest strength for onboarding devices securely. The RW612 supports Wi-Fi 6 and Bluetooth LE 5.3 (Zephyr Project, n.d.), useful for crowded home networks and phone-based setup.

Flash and RAM to Support the Code

My code is Python and depends on Linux, python-statemachine, gpiozero, and Adafruit's libraries, so only the Pi Zero 2 W, with 512 MB of RAM, runs it as written. The WFI32E01's 1 MB of flash and 256 KB of RAM is plenty for a C version of a three-state thermostat, but the program would need to be ported. The RW612's 1.2 MB of SRAM and external flash give the most headroom of the two microcontrollers for the Wi-Fi stack, TLS, and future over-the-air updates.

Recommendation

I recommend the Raspberry Pi Zero 2 W for the next phase. It meets all three requirements, costs under $20 (SparkFun Electronics, n.d.), and reuses code that is already tested, so the team can focus on cloud connectivity instead of a rewrite. The tradeoffs are higher power use, longer boot time, and a less durable microSD card. At high production volume, I would revisit the WFI32E01, since its built-in cloud security and lower unit cost would matter more at scale.

AI Usage Acknowledgment

I used a generative AI tool to help complete the Thermostat.py template, draft the state machine diagram, research specifications, and organize this report. I reviewed, tested, and revised all AI-assisted work.

References

Mouser Electronics. (n.d.). Microchip Technology WFI32E01 Wi-Fi microcontroller modules. https://www.mouser.com/new/microchip/microchip-wfi32e01-wifi-modules/

SparkFun Electronics. (n.d.). Raspberry Pi Zero 2 W. https://www.sparkfun.com/raspberry-pi-zero-2-w.html

Zephyr Project. (n.d.). NXP RD-RW612-BGA. https://docs.zephyrproject.org/latest/boards/nxp/rd_rw612_bga/doc/index.html